MD-PIPLINE – FERTICO2-AI

Technical Documentation for the Reconstruction of the Molecular Dynamics Database Used in the FertiCO₂-AI Artificial Intelligence Model

Project

FertiCO₂-AI: Artificial Intelligence Platform Applied to the Design of Biodegradable Ionic Liquids for CO₂ Capture and Green Fertilizer Production

Authors:

Guilherme Colherinhas de Oliveira; Wesley Bueno Cardoso; Leonardo Bruno Assis Oliveira; Tertius Lima da Fonseca; Karinna Mendanha Soares; Lucas de Sousa Silva; Henrique de Araujo Chagas

Institution:

Institute of Physics – Federal University of Goiás (UFG)

1. Objective of the Molecular Dynamics Pipeline

The molecular dynamics (MD) simulation pipeline developed within the scope of the FertiCO₂-AI project aims to generate high-quality physicochemical, structural, and dynamical data for the characterization of interactions between amino-acid-based ionic liquids (AAILs) and carbon dioxide (CO₂).

This pipeline constitutes the fundamental physical and scientific basis of the proposed solution, providing reliable molecular descriptors that support the subsequent stages of unsupervised classification and machine learning. All data used in the project are obtained exclusively from deterministic computational simulations, with no involvement of personal, sensitive, or human experimental data.

2. Computational Environment and Software Tools

All simulations are performed exclusively using open-source software, ensuring full reproducibility, auditability, and adherence to open science principles:

- Molecular Dynamics Engine: GROMACS (GNU GPL license)

- Force Field: CHARMM36

- Operating System: Linux (Ubuntu or equivalent)

- Execution: CPU, with optional OpenMP/MPI parallelization and GPU support

The computational environment is initialized using the standard GROMACS configuration command:

$ source /usr/local/gromacs/bin/GMXRC

3. Standardized Directory Structure

Each AAIL system is simulated independently following a standardized directory structure that ensures full traceability between chemical composition, simulation parameters, and generated data:

SYSTEM_NAME/
├── CONF-START.gro or CONF-START-CO2.gro
├── topol.top
├── index.ndx
├── charmm36.ff/
├── SCRIPT-MD or SCRIPT-MD-CO2
├── GROMPP-01.mdp to GROMPP-10.mdp
├── GROMPP-11.mdp and GROMPP-12.mdp
└── confout.gro

This organization guarantees that any system can be fully reconstructed from its base input files.

4. Stage 1 – Initial System Preparation

4.1 Initial Configuration

The initial configuration of each simulated system is defined in the file CONF-START.gro, which contains the three-dimensional coordinates of all molecules composing the amino-acid ionic liquid (AAIL) prior to energy minimization and molecular dynamics steps. This configuration represents either a pure ionic liquid or a binary mixture of AAILs, constructed to ensure global charge neutrality, correct cation–anion stoichiometry, and the absence of critical atomic overlaps that could compromise numerical stability.

Initial configurations are generated using the Packmol software, widely adopted for constructing condensed molecular systems based on geometric constraints and minimum interatomic distances. In the adopted protocol, amino-acid cations and anions are randomly inserted into a three-dimensional periodic box while respecting predefined minimum distances to avoid initial steric clashes and excessive repulsive forces during energy minimization. Initial box dimensions are chosen to allow subsequent volumetric relaxation during NPT equilibration.

System composition varies according to simulation objectives. For pure ionic liquids, approximately 300 ion pairs are employed, ensuring sufficient system size for statistically meaningful sampling of structural, dynamical, and energetic properties. For binary mixtures, different relative proportions of two distinct AAILs are considered, ranging from 50–250 to 250–50 ion pairs, while keeping the total number of ion pairs constant. This strategy enables the investigation of cooperative and competitive effects between different chemical species while preserving comparability across systems.

All initial configurations generated by Packmol undergo structural validation to ensure the absence of critical overlaps, incorrect bonding, or topological inconsistencies. The resulting CONF-START.gro file serves as a common starting point for all subsequent simulations, including energy minimization, NVT/NPT equilibration, and production runs, thereby ensuring pipeline standardization and full reproducibility.

4.2 Topology and Parameters

The complete molecular interaction description is defined in the main topology file (topol.top), which constitutes the physicochemical core of the molecular dynamics model. This file consolidates all information required for the accurate representation of amino-acid ionic liquids (AAILs), CO₂ molecules, and any additional system components, ensuring consistency, numerical stability, and physical fidelity throughout the simulations.

The CHARMM36 force field is employed, as it is extensively validated in the literature for complex molecular systems, including ionic liquids, polar media, and hydrogen-bond-dominated intermolecular interactions. CHARMM36 parameters are incorporated into topol.top via inclusion (.itp) files, providing a robust description of both bonded interactions (covalent bonds, angles, and dihedrals) and non-bonded interactions (Lennard-Jones potentials and long-range electrostatics).

The topology file explicitly defines atomic types, partial charges, and pairwise interaction parameters. Partial charges are consistent with the CHARMM36 force field and reflect the average electronic distribution of the molecular constituents, which is essential for accurately capturing Coulombic interactions between AAILs and CO₂. These definitions are critical for the reliable reproduction of interaction energies, hydrogen bond formation, and thermodynamic properties.

Additionally, topol.top hierarchically organizes system components, facilitating the definition of molecular groups later used in energetic, structural, and dynamical analyses. This organization ensures that the entire simulation and analysis pipeline remains fully reproducible, auditable, and extensible, in alignment with the open science principles adopted by the FertiCO₂-AI project.

5. Stage 2 – Equilibration of the Pure Ionic Liquid

This stage drives the system from its initial configuration to a thermodynamically stable state. The main execution script is SCRIPT-MD.

5.1 Equilibration Protocol

The equilibration protocol consists of the sequential execution of the files GROMPP-01.mdp through GROMPP-08.mdp, followed by 30 consecutive cycles of:

- Energy minimization

- Structural relaxation

This procedure ensures density convergence, stabilization of potential energy, and reorganization of the hydrogen-bond network.

5.2 Production Simulations

Files GROMPP-09.mdp and GROMPP-10.mdp are used to verify dynamical and energetic stability prior to CO₂ insertion.

5.3 Automated Execution

$ SCRIPT-MD

The entire process is automated to guarantee consistency across different systems.

6. Stage 3 – Explicit CO₂ Insertion

After full equilibration of the pure ionic liquid or binary AAIL mixture, the preparation of the AAIL–CO₂ system is performed. This step aims to construct a physically realistic initial configuration for the explicit study of CO₂ capture, adsorption, and molecular interactions.

The final equilibrated configuration (confout.gro) serves as the starting point. This file represents a structurally stabilized system with converged volume, density, and potential energy, ensuring that CO₂ insertion is not influenced by non-equilibrium artifacts.

6.1 Automated CO₂ Insertion Procedure

The AAIL–CO₂ system is prepared using a standardized automated procedure:

- Simulation cloning: The directory of the equilibrated ionic liquid simulation is cloned into a new directory following the naming pattern SYSTEM_NAME-CO2.

- Configuration conversion:

- gmx editconf -f confout.gro -o liquid.pdb

- Box dimension extraction:

- tail confout.gro

- Box expansion and CO₂ insertion: Using the Packmol_CO2.in input file, the ionic liquid is preserved at the center of the box while approximately 5 nm of free space is added along both sides of the Z-axis. A total of 300 CO₂ molecules are inserted into these regions, ensuring no critical overlaps.

Final conversion and box definition:

gmx editconf -f CONF-START-CO2.pdb -o CONF-START-CO2.gro -c -box X Y Z+10

Auxiliary file updates:

- Generation of a new index.ndx

- Update of topol.top to include 300 CO₂ molecules and the CHARMM36 CO2.itp

- Adjustment of GROMPP-12.mdp to allow explicit AAIL–CO₂ interaction energy calculations

Execution of CO₂ simulations:

$ SCRIPT-MD-CO2

This script performs an initial NVT relaxation followed by a production NVT simulation of up to 15 ns, storing approximately 30,000 configurations for statistical analysis.

7. Stage 4 – AAIL–CO₂ Production Simulations

7.1 Molecular Dynamics Simulations

- GROMPP-11.mdp

- GROMPP-12.mdp

These trajectories are used to compute average properties and statistical fluctuations.

7.2 Execution

$ SCRIPT-MD-CO2

8. Generated Data and Descriptor Extraction

The simulations automatically produce:

- Trajectory files (.trr, .xtc)

- Energy files (.edr)

- Log files (.log)

- Final configurations (confout.gro)

From these outputs, physicochemical, structural, and thermodynamic descriptors are extracted using standard GROMACS analysis tools:

- Interaction Energies (AAIL–CO₂)

  - gmx energy -f ener.edr -o energy_AAIL_CO2.xvg

  - Coulomb (SR) and Lennard-Jones (SR) contributions between AAIL and CO₂ groups are selected.

- Density

  - gmx energy -f ener.edr -o density.xvg

- Diffusion Coefficient (MSD)

  - gmx msd -f traj.xtc -s topol.tpr -n index.ndx -o msd.xvg

- Hydrogen Bond Statistics

  - Number of hydrogen bonds:

    - gmx hbond -f traj.xtc -s topol.tpr -n index.ndx -num hbnum.xvg

  - Lifetime (Luzar–Chandler autocorrelation):

    - gmx hbond -f traj.xtc -s topol.tpr -n index.ndx -ac hbac.xvg

Hydrogen bond dissociation energy is estimated from the Luzar–Chandler formalism using parameters derived from the autocorrelation decay.

9. Reproducibility and Open Science Guarantees

The MD pipeline ensures:

- Deterministic and versioned protocols

- Exclusive use of open-source software

- Automated execution scripts

- Modular and standardized organization

- Full data regeneration capability

Any researcher can fully reproduce all simulations using the provided files and GROMACS alone.

10. Integration with the AI Platform

The data generated by this MD pipeline directly feed the FertiCO₂-AI artificial intelligence platform while remaining conceptually independent from the AI model itself. This separation ensures scientific traceability, independent data reuse, and full methodological transparency.